assignment #01

Course:            		CHE 433:  Process Simulation with Aspen Plus

Term:			Fall   2025

Professor:    		Dr. Michael Caracotsios

Office:			EIB 244

Cell Phone:		(630) 696-0934

E-mail:           		mcaracot@uic.edu

Student names: ______________________________________

Due Date:      Monday, September 8th in Class (3:30PM)

INSTRUCTIONS FOR ASSIGNMENT DELIVERY

- Prepare a typed report of your assignment.

- Include this cover page with your name(s) in the report.

- Upload your report as a PDF file by the Due Date and Time on the UIC Blackboard.

- Bring a hard copy of your report as a PDF file by the Due Date and Time in class

- Assignments that are not typed will not be accepted.

- Late Assignments will not be accepted without prior approval.

- You may use the Internet as well as ChatGPT or other public tools as long as you place a written statement about their use in your assignment report

# Problem A:  Review of Chemical Engineering Fundamentals

Your first assignment after six months of training in the UIC Specialty Chemicals Corporation is to provide a material balance for a cyclohexane production unit and preliminary cost estimates of the utilities. A simple process flow diagram (shown in Figure A below) has been given to you by a mid-level engineer who has decided to leave the company. The data for this process are given below:

Benzene (BZ) Feed

Flow: 92.14 lbmole/hr

Temperature: 100 oF

Pressure: 15 psia

Hydrogen (H2) Feed

Flow: 283.8 lbmole/hr

Temperature: 120 oF

Pressure: 335 psia

H2 mole fraction : 0.975

N2 mole fraction :  0.005

CH4 mole fraction : 0.020

Charge Heater

Temperature: 300 oF

Pressure: 330 psia

Reactor

Temperature: 400 oF

Pressure: 315 psia

Benzene Conversion: 99.86%

H2 to BZ molar ratio: 3.3

High Pressure (HP) Product Separator (Flash Drum)

Temperature: 120 oF

Pressure : 300 psia

Low Pressure (LP) Product Separator (Flash Drum):

Pressure : 15 psia

Your tasks

- Complete the process flowsheet by adding the missing unit operations and provide a brief explanation for every unit you add.

- Prepare the material balance in the format of the ENG Calculations of the CHE Calculator. Provide a brief explanation on how you calculate the effluent streams from each unit operation. Additionally provide a brief description of the numerical method(s) you have used to derive the material balance

- Provide a preliminary cost of all utilities

- Propose a method to reduce the utilities cost and do a simulation to calculate the cost reduction of your proposal

Data and Assumptions:

- You may assume that the hydrocarbon mixtures with hydrogen presence in the process follow the Peng-Robinson equation of state

- You may also assume that the K-values for the flash calculations can be calculated from Raoult’s law or the Wilson equation

- The fuel in the charge heater is natural gas

- Utility cost data can be obtained from a variety of sources

Figure A: Hydrogenation of Benzene to Cyclohexane