Student Name:

CHE 230: Molecular Systems in Chemical Engineering (CHE 230)

Quiz 2

- Which of these is not a type of point defect

- Vacancy

- Point dislocation

- Substitutional Impurity

- Self-Interstitial

- Examining the variables in Fick's First Law (J= -DdCadx). Which of the following changes will directly cause the value of the diffusion coefficient to change?

- Increasing the Concentration Gradient (dCadx).

- Increasing the Temperature (T) of the system .

- Increasing the Diffusion Time (t).

- Increasing the cross-sectional area (A).

- Point defect matters for these reasons, except

- Enable atomic diffusion.

- Influence mechanical strength.

- Control electrical conductivity.

- Creating long-range plastic deformation.

- Which of the following correctly ranks the Activation Energy (Q) required for atomic diffusion through different paths, from highest to lowest?

QL = Activation Energy for Lattice Diffusion

QGB = Activation Energy for Grain Boundary Diffusion

QS = Activation Energy for Surface Diffusion

- QL> QGB>QS

- QS >QGB >QL

- QGB >QL> QS

- QL>QS> QGB

- A metal bar is bent until it undergoes plastic deformation (permanent shape change). If the external force is completely removed, what happens to the bar?

- It remains exactly at the bent shape it held while the force was applied.

- It springs back slightly towards its original shape, but retains most of the deformation.

- It slowly returns to its original shape over time due to relaxation.

- It fractures immediately because the yield point was exceeded.

- In a tensile test, how is True Stress defined differently from Engineering Stress?

- True Stress is calculated using the instantaneous cross-sectional area ,while Engineering Stress uses the original area.

- True Stress is calculated using the original cross-sectional area ,while Engineering Stress uses the instantaneous area.

- True Stress is the natural logarithm of the instantaneous length divided by the original length.

- True Stress is the change in length divided by the original length.

- Plastic deformation in a crystalline metal primarily occurs because:

- Entire atomic planes slide over one another simultaneously.

- Individual atoms diffuse to new positions.

- Dislocations move through the crystal under applied stress.

- Grain boundaries fracture under load.

- Which statement correctly distinguishes an edge dislocation from a screw dislocation?

- Edge dislocations involve shear stress, while screw dislocations involve tensile stress.

- The Burgers vector is perpendicular to the dislocation line for an edge dislocation.

- Screw dislocations can only exist in polycrystalline materials.

- Edge dislocations have no associated stress field.

- ________