Student Name: _____________________________________________

CHE 230: Molecular Systems in Chemical Engineering (CHE 230)

Quiz 3

Date: 03-16-2026

1.  What is a phase?

a.  A type of chemical bond formed between two atoms in a molecule

b.  A distinct and homogeneous form of matter with uniform physical and chemical properties throughout

c.  The process by which a substance changes temperature during heating

d.  A unit used to measure the pressure of a gas in a closed system

2.  Which of these is not a 2-phase system?

a.  Pure water at 25°C and 1 atm

b.  A mixture of oil and water

c.  Ice and liquid water coexisting at 0°C and 1 atm

d.  Mild Steel

3.  A binary system at equilibrium has a liquid phase with 20 mol% component A and a vapor phase with 80 mol% component A. The overall composition is 50 mol% component A. Using the lever rule, what fraction of the system is in the liquid phase?

a.  0.25

b.  0.50

c.  0.75

d.  0.80

4.  What is a Eutectic System?

a.  A system where two substances are completely immiscible in all proportions

b.  A mixture of two or more components that has a single lowest melting point, where all components solidify simultaneously

c.  A system that undergoes a phase change only under high pressure conditions

d.  A type of polymer blend that expands when cooled below room temperature

5.  Which of these has the highest heat capacity?

a.  Aluminum

b.  Iron

c.  Water

d.  Copper

6.  Which of these is FALSE about thermal conductivity?

a.  Metals generally have higher thermal conductivity than non-metals

b.  Thermal conductivity increases with temperature in all materials

c.  Gases have lower thermal conductivity than liquids and solids

d.  Thermal conductivity is a material property that describes heat transfer rate

7. Which material would most likely behave as an insulator?

A. A material with overlapping energy bands
B. A material with a partially filled band
C. A material with a very large band gap
D. A material with a small band gap

8. Which property primarily determines whether a material can exhibit magnetism?

A. Atomic mass
B. Number of protons
C. Presence of unpaired electrons
D. Density

9. In antiferromagnetic materials, neighboring spins are:

A. Randomly oriented
B. Parallel
C. Opposite and equal
D. Opposite but unequal

10. A material appears white under white light. Which wavelengths of visible light does it most likely reflect?

A. Only red wavelengths
B. Only blue wavelengths
C. All visible wavelengths approximately equally
D. No wavelengths

Extra Credit

1.  During class, the professor spoke about a particular experiment during his research that required heating. What was the composition of the heating system?

________________________  and  ________________________